EW 1 LAB 7

Rhea Bagchi 2023102023

Snigdha stp 2023102036

Name of the Experiment: Android-Arduino Integration

Aim:

- Controlling LED on Arduino uno with App developed in MIT APP Inventor using Bluetooth module

- Reading values from sensor connected to Arduino uno and display them on the Android App developed in MIT APP Inventor using Bluetooth module.

Components required:

- Computer with internet access

- Android smartphone or tablet

- Arduino

- HC-05 Bluetooth module

- Account of MIT app inventor in the device

- LED, breadboard, wires.

Procedure:

Make arduino connections: 10- Rx , 11 – Tx, ledpin -8, pingpin – 13, echopin – 12.

- Write the code to use sensor on arduino IDE.

- Use HC-05 bluetooth module to upload code to MIT app inventor.

- Once connected, use the ON\OFF switches, and place objects in front of sensor.

Controlling LED on Arduino uno with MIT app inventor:

Arduino code for the experiment:

#include SoftwareSerial mySerial(10,11); // 10- Rx , 11 - Tx

int ledpin=13;

int Data;

void setup()

{

mySerial.begin(9600);

pinMode(ledpin,OUTPUT);

}

void loop() {

if (mySerial.available() )

{

Data=mySerial.read();

if(Data=='1')

{ digitalWrite(ledpin,HIGH);

mySerial.println("LED On! ");

}

else if (Data=='0')

{ digitalWrite(ledpin,LOW);

mySerial.println("LED Off! ");

}

}

}

Observations:

Read values from sensor connected to Arduino uno and display them on the Android App developed in MIT APP Inventor using Bluetooth module:

Arduino code for the experiment:

#include <SoftwareSerial.h>

SoftwareSerial mySerial(0, 1);  // 10- Rx , 11 - Tx

int ledpin = 8 ;

int Data, k = 0;

const int pingPin = 13;  // Trigger Pin of Ultrasonic Sensor

const int echoPin = 12;  // Echo Pin of Ultrasonic Sensor

float duration, cm,distance_cm;

void setup() {

mySerial.begin(9600);

pinMode(ledpin, OUTPUT);

pinMode(pingPin, OUTPUT);

pinMode(echoPin, INPUT);

}

void loop() {

if (mySerial.available()) {

Data = mySerial.read();

if (Data == '1') {

digitalWrite(ledpin, HIGH);

digitalWrite(pingPin, HIGH);

delayMicroseconds(10);

digitalWrite(pingPin, LOW);

duration = pulseIn(echoPin, HIGH);

// cm = microsecondsToCentimeters(duration);

distance_cm = 0.017 * duration;

mySerial.print("distance: ");

mySerial.print(distance_cm);

mySerial.println(" cm");

delay(200);

} else if (Data == '0') {

digitalWrite(ledpin, LOW);

}

}

}

Observations:

Conclusion:

- Sensor with HC-05 bluetooth connector and MIT app inventor can be used to find displacement of object from sensor.